HW 9-10 Soln)

Forces on the wheel:  weight, force for the axle, and the tension.

Forces on the hanging mass: weight and tension.

Wg – conservative

Waxle force – no displacement of the wheel, so no work done

WT – although the tension does positive work on the wheel and negative work on the hanging mass, these terms exactly cancel to zero.

Let the hanging mass start at y = 0.

So, WNC = 0.  Then,

0= 12mvf2-12mvi2+12Mvf2-12Mvi2+12Iωf2-12Iωi2+gmyf-gmyi+gMyf-gMyi

The terms in red are either zero of exhibit no change over the duration of the problem.

0= 12mvf2+12Iωf2+gmyf

Similarly to the rolling without slipping problems, the wheel lets out string such that its angular speed is correlated to the translational sped of the hanging mass:

v=Rω  .

Then,

0= 12mvf2+1212MR2vfR2+gmyf

12m+14Mvf2=-gmyf

vf=-gmyf12m+14M= -100.5(-3)120.5+141.8=4.63 m/s